EE4620L/EE6620L/CEG4324L/CEG6324L

DIGITAL INTEGRATED CIRCUIT DESIGN LAB

Lab 3

By

Yeoh_________ Alex________________

UID: U00931845

Email: yeoh.3@wright.edu

Submission date: 01/06/2024

“I have neither given nor received aid on this assignment, nor have I observed any violation of the Honor code”

Signature : Alex Yeoh

Date : 31/05/2024

Table of Contents (include page #)

I. Introduction (page 4)

II. Process (page 4)

III. Results (page 4)

IV. Conclusion (page 8)

V. Code (page 9)

List of Figures

Figure 1: booth multiplier stage design in Logisim. (page 4)

Figure 2: booth multiplier simulation with the inputs and outputs in signed decimal. (page 7)

Figure 3: booth multiplier simulation with the inputs and outputs in hexadecimal. (page 7)

Figure 4: booth multiplier simulation with the inputs and outputs in binary. (page 7)

Figure 5: area, power, and delay analysis for the booth multiplier (page 7)

Figure 6: schematic for the booth multiplier (page 8)

Figure 7: Zedboard with the booth multiplier performing 7 * 21. (page 8)

I. Introduction

The objective of the lab is to become more familiar with Vivado by building and simulating a booth multiplier.

II. Process

I first designed a booth multiplier in logisim to determine the required logical components with the design shown below in figure 1.

Figure 1: booth multiplier stage design in Logisim.

I then made the Vivado project and imported all the given files. I then replicated the booth multiplier design in booth.vhd. I then fixed tb_booth.vhd to test the four test cases. Finally, I demonstrated the booth multiplier design working on the Zedboard.

III. Results

The following are the results for the booth multiplier.

The hand computed results for each of the four cases.

7 * 21 a: 00000111 b: 00010101 x: 000000000010101 p: 000000000000000 append 0 to a a: 000001110 First operation 110: p = p + -x 000000000000000 + 111111111101011 p: 111111111101011 x: 000000001010100 Second operation 011: p = p + 2x 111111111101011 + 000000010101000 p: 000000010010011 x: 000000101010000 Third operation 000: p = p + 0 000000010010011 + 000000000000000 p: 000000010010011 x: 000010101000000 Forth operation 000: p = p + 0 000000010010011 + 000000000000000 p: 000000010010011 x: 001010100000000 Output: 000000010010011 = 147

7 * -31 a: 00000111 b: 11100001 x: 111111111100001 p: 000000000000000 append 0 to a a: 000001110 First operation 110: p = p + -x 000000000000000 + 000000000011111 p: 000000000011111 x: 111111110000100 Second operation 011: p = p + 2x 000000000011111 + 111111100001000 p: 111111100100111 x: 111111000010000 Third operation 000: p = p + 0 111111100100111 + 000000000000000 p: 111111100100111 x: 111100001000000 Forth operation 000: p = p + 0 111111100100111 + 000000000000000 p: 111111100100111 x: 110000100000000 Output: 111111100100111 = -217

-31 * -31 a: 11100001 b: 11100001 x: 111111111100001 p: 000000000000000 append 0 to a a: 111000010 First operation 010: p = p + x 000000000000000 + 111111111100001 p: 111111111100001 x: 111111110000100 Second operation 000: p = p + 0 111111111100001 + 000000000000000 p: 111111111100001 x: 111111000010000 Third operation 100: p = p + -2x 111111111100001 + 000001111100000 p: 000001111000001 x: 111100001000000 Forth operation 111: p = p + 0 000001111000001 + 000000000000000 p: 000001111000001 x: 110000100000000 Output: 000001111000001 = 961

-31 * 21 a: 11100001 b: 00010101 x: 000000000010101 p: 000000000000000 append 0 to a a: 111000010 First operation 010: p = p + x 000000000000000 + 000000000010101 p: 000000000010101 x: 000000001010100 Second operation 000: p = p + 0 000000000010101 + 000000000000000 p: 000000000010101 x: 000000101010000 Third operation 100: p = p + -2x 000000000010101 + 111110101100000 p: 111110101110101 x: 000010101000000 Forth operation 111: p = p + 0 111110101110101 + 000000000000000 p: 111110101110101 x: 0010101000000 Output: 111110101110101 = -651

Figure 2: booth multiplier simulation with the inputs and outputs in signed decimal.

Figure 3: booth multiplier simulation with the inputs and outputs in hexadecimal.

Figure 4: booth multiplier simulation with the inputs and outputs in binary.

Figure 5: area, power, and delay analysis for the booth multiplier

Figure 6: schematic for the booth multiplier

Figure 7: Zedboard with the booth multiplier performing 7 * 15.

IV. Conclusion

From this lab, I have learned how to write more complicated VHDL code such as a booth multiplier.

V. Code

-- Booth Multiplier -- Define libraries library ieee; use ieee.std_logic_1164.all; use ieee.numeric_std.all; use ieee.std_logic_unsigned.all; entity booth is port (operand_a : in std_logic_vector (7 downto 0); operand_b : in std_logic_vector (7 downto 0); rst : in std_logic; clk : in std_logic; z : out std_logic_vector (14 downto 0)); end booth; library ieee; use ieee.std_logic_1164.all; use ieee.numeric_std.all; use ieee.std_logic_unsigned.all; entity stage is port (y : in std_logic_vector (2 downto 0); xin, pin: in std_logic_vector (14 downto 0); overlap: out std_logic; xout, pout: out std_logic_vector (14 downto 0)); end stage; architecture name of stage is signal op:std_logic_vector(1 downto 0); signal localx,localx1,localx2:std_logic_vector(14 downto 0); begin -- overlap bit so booth doesn't need to take from the overall instruction overlap <= y(2); -- left shift 2 for next stage xout <= xin(12 downto 0) & "00"; -- making signal for choosing 0, x, or 2x op(0) <= y(0) when (y(2)='0')else not y(0) after 2 ns; --1 inverter op(1) <= y(1) when (y(2)='0')else not y(1) after 2 ns; --1 inverter -- deciding between 0 or x localx <= xin when ((op(0) or op(1))='1') else "000000000000000" after 4 ns; --2in or gate -- multiplying by 2 by bit shifting, only relevant when previous step decided x localx1 <= localx when ((op(0) and op(1))='0') else localx(13 downto 0) & '0' after 4 ns; --2in and gate -- effectively xor localx to make adder into an adder/subtractor localx2 <= localx1 when (y(2)='0')else not localx1 after 30 ns; --15 inverters -- adder pout <= pin + localx2 + y(2); end; architecture rtl of booth is --Declare Signals signal x_ext, xout0, xout1, xout2, pout0, pout1, pout2: std_logic_vector (14 downto 0); signal overlap: std_logic_vector (2 downto 0); --Declare Components component stage is port (y : in std_logic_vector (2 downto 0); xin, pin: in std_logic_vector (14 downto 0); overlap: out std_logic; xout, pout: out std_logic_vector (14 downto 0)); end component; begin x_ext <= operand_b(7) & operand_b(7) & operand_b(7) & operand_b(7) & operand_b(7) & operand_b(7) & operand_b(7) & operand_b; stg1: stage port map (y(0) => '0', y(1) => operand_a(0), y(2) => operand_a(1), xin => x_ext, pin => "000000000000000", overlap => overlap(0), xout => xout0, pout => pout0); stg2: stage port map (y(0) => overlap(0), y(1) => operand_a(2), y(2) => operand_a(3), xin => xout0, pin => pout0, overlap => overlap(1), xout => xout1, pout => pout1); stg3: stage port map (y(0) => overlap(1), y(1) => operand_a(4), y(2) => operand_a(5), xin => xout1, pin => pout1, overlap => overlap(2), xout => xout2, pout => pout2); stg4: stage port map (y(0) => overlap(2), y(1) => operand_a(5), y(2) => operand_a(7), xin => xout2, pin => pout2, pout => z); end;

booth.vhd

library IEEE; use IEEE.STD_LOGIC_1164.ALL; use std.env.stop; entity tb_booth is generic(tclk:time := 1 ns; -- clock speed WPD: time := 169 ns -- rate at which the input changes ); end tb_booth; architecture Behavioral of tb_booth is signal clk : std_logic; signal rst : std_logic; signal A : std_logic_vector (7 downto 0); signal B : std_logic_vector (7 downto 0); signal P : std_logic_vector (14 downto 0); component booth is port (operand_a : in std_logic_vector (7 downto 0); operand_b : in std_logic_vector (7 downto 0); rst : in std_logic; clk : in std_logic; z : out std_logic_vector (14 downto 0)); end component; begin clock:process begin clk <= '1'; wait for tclk / 2; clk <= '0'; wait for tclk / 2; end process; DUT: booth port map(clk=>clk,rst=>rst,operand_a=>A,operand_b=>B,z=>P); test_vectors:process begin A<="00000111"; --(7) B<="00010101"; --(21) rst<='1'; wait for WPD; rst<='0'; wait for WPD; A<="00000111"; --(7) B<="11100001"; --(-31) rst<='1'; wait for WPD; rst<='0'; wait for WPD; A<="11100001"; --(-31) B<="11100001"; --(-31) rst<='1'; wait for WPD; rst<='0'; wait for WPD; A<="11100001"; --(-31) B<="00010101"; --(21) rst<='1'; wait for WPD; rst<='0'; wait for WPD; stop; end process; end Behavioral;

tb_booth.vhd